Lesson_3_Notes_and_Journal

2024.04.23

- Look at Code

- Look at Circuit

- Draw Circuit

- Build Circuit

- Start Lesson

Code

- Glance at Code

import machine

import time

# Define LED Pin

LED_PIN = 17

led = machine.Pin(LED_PIN, machine.Pin.OUT)

# Define ADC Pin for LDR

ADC_PIN = 26

adc = machine.ADC(machine.Pin(ADC_PIN))

# Define a threshold for the LDR reading below which the LED should turn ON

LIGHT_THRESHOLD = 50000  # This might need adjustment based on your LDR and environment

def night_light():

while True:

light_value = adc.read_u16()

print(light_value)

# If the reading is below the threshold, turn the LED on. Otherwise, turn it off.

if light_value < LIGHT_THRESHOLD:

led.value(1)

else:

led.value(0)

time.sleep(0.5)  # Check every half second

night_light()

Circuit

- Theirs

Red--2

Red--2

Black-0

Black--0

Silver ± 10%

220 Ω

Brown --1

Black--0

Black--0

Red--00

Silver ± 10%

10,000 Ω  10K Ω

- Mine

- As built in video

4/25/24 Rewired to video

Now on when bright – Numerical Value is 17xxx to 20xxx

And off when I cover it with my finger --  59xxx to 62xxx

Room light on vs off doesn’t change values!

Reversed code from < to >