2024_05_03_Troubleshooting

Plan:

Use Parts of Binary Counter Circuit and Code that works to try to get this working.

Lesson_4_Troubleshooting_Notes_and_Journal

2024.05.03

- Look at Code

- Look at Circuit

- Use Lesson 6 Circuit

- Get Blue wire LED working

- Pare away unused LEDS, wires and resisters

- Re-Draw Circuit

- Test

Code

- First Revision

- # Define GPIO pins for the LED and button – 5/3/2024 Based on Lesson 4 Circuit

- led_pin = machine.Pin(3, machine.Pin.OUT) # WAS 14

- button_pin = machine.Pin(8, machine.Pin.IN, machine.Pin.PULL_DOWN) # WAS 15

- Second Revision

- Original Lesson 4 Glance at Code

- import machine

- import time

- # Define GPIO pins for the LED and button

- led_pin = machine.Pin(14, machine.Pin.OUT)

- button_pin = machine.Pin(15, machine.Pin.IN, machine.Pin.PULL_DOWN)

- # Function to measure reaction time

- def reaction_time_tester():

- time.sleep(5)

- led_pin.on()  # Turn on the LED

- start_time = time.ticks_ms()  # Get start time in milliseconds

- # Wait for button press

- while True:

- if button_pin.value() == 1:

- end_time = time.ticks_ms()  # Get end time in milliseconds

- reaction_time = end_time - start_time  # Calculate reaction time

- print('Reaction Time:', reaction_time, 'ms')

- break

- led_pin.off()  # Turn off the LED

- # Call the function to start the reaction time tester

- reaction_time_tester()

- Lesson 6 REFERENCE Glance at Code

import machine

import time

# Define GPIO pins for the 4 LEDs

led_pins = [

machine.Pin(0, machine.Pin.OUT),

machine.Pin(1, machine.Pin.OUT),

machine.Pin(2, machine.Pin.OUT),

machine.Pin(3, machine.Pin.OUT)

]

button_pin = machine.Pin(8, machine.Pin.IN, machine.Pin.PULL_DOWN)

# Initialize LED states for 4 LEDs

led_states = [0] * len(led_pins)

# Function to update LED states

def update_led_states():

for i, led in enumerate(led_pins):

if led_states[i] == 1:

led.on()

else:

led.off()

# Function to create a padded binary string

def padded_binary_string(value, length):

binary_str = bin(value).replace("0b", "")

while len(binary_str) < length:

binary_str = '0' + binary_str

return binary_str

# Function to increment binary counter

def increment_counter():

# Convert LED states to integer

binary_value = int(''.join(map(str, led_states)), 2)

binary_value += 1

# If value is 16 (10000 in binary), reset to 0

if binary_value >= 16:

binary_value = 0

binary_string = padded_binary_string(binary_value, len(led_pins))

# Update LED states

for i, bit in enumerate(binary_string):

led_states[i] = int(bit)

# Main loop

while True:

if button_pin.value() == 1:

increment_counter()

update_led_states()

time.sleep(0.2)  # Button debounce delay

Circuit

- Theirs

Blue Wire is Fifth Pin:  5 | GP3, etc.

Yellow Wire is Eleventh Pin: 11 | GP8, etc.