Lesson_4_Notes_and_Journal

2024.04.26

- Look at Code

- Look at Circuit

- Draw Circuit

- Build Circuit

- Start Lesson

Code

- Glance at Code

- import machine

- import time

- # Define GPIO pins for the LED and button

- led_pin = machine.Pin(14, machine.Pin.OUT)

- button_pin = machine.Pin(15, machine.Pin.IN, machine.Pin.PULL_DOWN)

- # Function to measure reaction time

- def reaction_time_tester():

- time.sleep(5)

- led_pin.on()  # Turn on the LED

- start_time = time.ticks_ms()  # Get start time in milliseconds

- # Wait for button press

- while True:

- if button_pin.value() == 1:

- end_time = time.ticks_ms()  # Get end time in milliseconds

- reaction_time = end_time - start_time  # Calculate reaction time

- print('Reaction Time:', reaction_time, 'ms')

- break

- led_pin.off()  # Turn off the LED

- # Call the function to start the reaction time tester

- reaction_time_tester()

Circuit

- Theirs

Brown 1

Black  0

Black 0

Red 2

Silver? ± 10%

10000 = 10 K Ω ± 10%

???  Red 2

Red 2

Black 0

Black 0

Silver? ± 10%

220 Ω ± 10%

Black – GND

Orange –  GP14 -- LED

Yellow – GP15 -- Button

Red –    36 3V3 (OUT)

- Mine

Original from Web Site

Corrected

- As built